09.07.2025

# THESIS PROGRESS REPORT

Middle East Technical University (METU)

Graduate School of Natural and Applied Sciences

Electrical and Electronics Engineering

AI-Powered Decision Support System for Wildfire Disaster Response and Management (DISASTERAWARE)

Çağlar AKMAN

Supervisor: Prof. M. Kemal LEBLEBICIOĞLU

# Table of Contents

Table of Contents	1

List of Figures	3

List of Tables	4

Acronyms & Definitions	5

Nomenclature	6

1	Executive Summary	8

2	Introduction	10

3	System Architecture	12

4	Technical Background	14

4.1	Review of Wildfire Simulation Models	14

4.2	Rothermel Based Wildfire Modelling	16

4.3	Fuzzy Logic in DSS	17

5	Wildfire Simulation with DSS Integration	19

5.1	Algorithmic Flow	19

5.2	Simulation Input Parameters	22

5.3	Uncertainty of Simulation Input Parameters	25

5.4	Simulation Outputs	27

5.5	Cost Function and Optimization Objective	30

6	DSS Decision-Making Framework	33

6.1	Observation Space Definition	33

6.2	Action Set and Decision Categories	35

6.2.1	Action Set Structure	35

6.3	Fuzzy Logic Architecture	38

6.4	Rule Set Examples	38

7	Conclusion	39

8	Appendices	40

8.1	Appendix A: Fully Detailed Pseudocode for Spatial-Temporal Wildfire Simulation	40

8.1.1	Initialization Phase	40

8.1.2	Ignition Setup	40

8.1.3	Main Simulation Loop	40

8.1.4	Update Cell States & Resources	41

8.1.5	Output & Advance Time	42

8.1.6	Cost Function Evaluation (Periodically)	42

9	References	43

# List of Figures

Figure 1 Overall System Diagram	12

Figure 2 Example of Triangular Membership Functions	18

Figure 3 θfirevs θwind and θslope for ws=ww=0.5	21

Figure 4 fθfire,θnbr=max0,cosθfire-θnbr vs θfire,θnbr	21

# List of Tables

Table 1 Gap Analysis	10

Table 2 SoA Models and their key features with limitations	14

Table 3 Comprehensive Comparative Table of Fire Simulation Models	15

Table 4 Rothermel model equations and explanations	16

Table 5 Simulation Steps	19

Table 6 Simulation input parameters by category	22

Table 7 Fuel-Dependent Parameter Lookup Table	24

Table 8 Categorization of Uncertainty Sources	25

Table 9 Simulation output	27

Table 10 Weights of parameters for Cost Function	30

Table 11 Observation Space Parameter List	33

Table 12 Intervention Type Categories	35

Table 13 Timing Options	37

Table 14 Scale Options	38

# Acronyms & Definitions

| Abbreviation |  | Definition |
| --- | --- | --- |
| AI | : | Artificial Intelligence |
| BEHAVE | : | Fire Behavior Prediction and Fuel Modelling System |
| CV | : | Coefficient of Variation |
| DEM | : | Digital Elevation Model |
| DSS | : | Decision Support System |
| FARSITE | : | Fire Area Simulator |
| FB | : | Fire Behavior |
| FBP | : | Forest Fire Behavior Prediction |
| FIRETEC | : | Fire Science-Based Wildfire Simulation Tool |
| FSPro | : | Fire Spread Probability |
| GIS | : | Geographic Information System |
| MTT | : | Minimum Travel Time |
| RH | : | Relative Humidity |
| SoA | : | State of the Art |
| UI | : | User Interface |
| USFS | : | United States Forest Service |
| WFDS | : | Wildland-Urban Interface Fire Dynamics Simulator |
| WUI | : | Wildland-Urban Interface |

# Nomenclature

| Symbol |  | Description | Units |
| --- | --- | --- | --- |
| Aburned | : | Area burned | m² |
| Arisk | : | Area at risk | m² |
| B | : | Wind coefficient factor – exponent | - |
| C | : | Wind coefficient factor – linear | - |
| Cair | : | Aerial resource unit cost | USD/unit/hour |
| Carbem | : | Carbon emission | kg/m² |
| Ceco | : | Economic value per unit area | USD/m² |
| Cgnd | : | Ground resource unit cost | USD/unit/hour |
| Ctotal | : | Total suppression cost | USD |
| CV | : | Coefficient of variation | - |
| D | : | Population density | persons/m² |
| d | : | Side length of simulation cell | m |
| E | : | Wind coefficient factor – wind modification | - |
| Efactor | : | Emission factor | kg_emission/kg_fuel |
| h | : | Heat of combustion | kJ/kg |
| I | : | Reaction intensity | kJ/min·m² |
| Iinit | : | Initial reaction intensity | kJ/min·m² |
| Ith | : | Threshold reaction intensity (burnout) | kJ/min·m² |
| IE | : | Intervention effectiveness | % |
| J | : | DSS cost function value | - |
| L | : | Flame length | m |
| M | : | Fuel moisture content | % |
| M_ext | : | Extinction moisture content | % |
| N | : | Number of burned cells | - |
| P | : | Population | persons |
| Qig | : | Heat of preignition | kJ/kg |
| R | : | Fire spread rate | m/min |
| Rair | : | Aerial resource amount | kg or lt |
| Rgnd | : | Ground resource amount | kg or lt |
| Rinit | : | Initial resource amount | kg or lt |
| RH | : | Atmospheric relative humidity | % |
| Sair | : | Aerial suppression rate | kW/m²/min |
| Sgnd | : | Ground suppression rate | kW/m²/min |
| T | : | Atmospheric temperature | °C |
| Tburn | : | Burn duration | min |
| Tign | : | Ignition time | min |
| Top | : | Operation duration | min |
| Tspread | : | Time to spread to adjacent cell | min |
| U | : | Wind speed | km/h |
| Vair | : | Aerial resource speed | km/h |
| Vgnd | : | Ground resource speed | km/h |
| W | : | Fuel loading | kg/m² |
| Wres | : | Residual fuel load | kg/m² |
| Winit | : | Initial fuel loading | kg/m² |
| α | : | Adaptation coefficient (learning rate) | - |
| β | : | Packing ratio | - |
| βopt | : | Optimal packing ratio | - |
| δ | : | Fuel bed depth | m |
| ϵ | : | Effective heating number | - |
| ηM | : | Moisture damping coefficient | - |
| ηs | : | Mineral damping coefficient | - |
| Γ | : | Maximum reaction velocity | 1/min |
| σ | : | Surface area-to-volume ratio | 1/m |
| θfire | : | Direction of fire spread | rad |
| θneighbor | : | Direction to neighbor cell | rad |
| θslope | : | Slope direction | rad |
| θwind | : | Wind direction | rad |
| ϕs | : | Slope coefficient | - |
| ϕw | : | Wind coefficient | - |
| w1…w8 | : | Cost function weights | - |
| ws | : | Weighting factor for slope influence | - |
| ww | : | Weighting factor for wind influence | - |
| XYaerial | : | Aerial reachable region | - |
| XYgnd | : | Ground reachable region | - |

# Executive Summary

This thesis progress report presents the design, development, and evaluation of DISASTERAWARE, an AI-powered Fuzzy Logic-based Decision Support System (DSS) for wildfire disaster response and management. The system aims to optimize intervention strategies by integrating real-time simulation data, uncertainty modelling, and intelligent decision-making mechanisms into a coherent, modular framework.

The foundation of this work is built upon a comprehensive review of wildfire simulation models. Multiple methodologies including empirical, semi-empirical, and physics-based approaches were examined and compared based on their accuracy, computational efficiency, and operational relevance. The Rothermel model (1972) was identified as the most suitable base model due to its balance between computational efficiency and reliability. While physics-based models such as FIRETEC and WFDS offer high fidelity, their excessive computational cost limits their real-time usability. Therefore, the DISASTERAWARE system is constructed upon a Rothermel-based simulation framework, integrated with additional stochastic and decision-making layers.

The proposed system architecture includes interconnected modules such as the Simulation Engine, Grid Map, Decision Support System, and a Cost Function Optimizer. The simulation engine operates on a cell-based grid where wildfire spread is calculated at each timestep using Rothermel’s equations, incorporating wind, slope, fuel characteristics, and moisture content. Dynamic attributes such as ignition status, fire direction, and cell transitions (Unburned → Burning → Burned) are continuously updated. Resource movements (aerial and ground) and population data are integrated at the cell level to simulate real-time interventions and risks.

To address the variability and uncertainty inherent in wildfire environments, a detailed stochastic modelling approach is applied. Environmental inputs like wind speed, temperature, fuel load, and suppression rates are treated as random variables with empirically justified distributions (e.g., Normal, Triangular, Uniform). The system runs ensemble simulations, enabling statistical aggregation of outputs (mean, standard deviation) and computing the Coefficient of Variation (CV) for each performance metric. A combined Prediction Confidence Score is derived from these CVs, reflecting the reliability of each simulation episode and influencing the DSS response strength.

The Decision Support System employs a fuzzy rule-based inference engine. It receives updated observation space variables (fire spread rate, resource availability, intervention delays, etc.) and suggests the most appropriate action from a pre-defined action set (e.g., deploy ground units, redirect aerial assets, delay intervention). The fuzzy logic system is designed with expert-defined membership functions, rule sets, and defuzzification procedures, and adapts its aggressiveness based on prediction confidence levels.

A multi-dimensional cost function lies at the core of decision evaluation. This cost function includes variables, namely burned area, economic loss, suppression cost, affected population, emissions, residual fuel load, intervention effectiveness, prediction confidence. Each term is weighted based on literature guidance and updated dynamically through a learning-based adaptation rule. The DSS continuously seeks to minimize this cost, thereby guiding operational decisions under real-time and uncertain wildfire conditions.

In conclusion, this study contributes a novel, integrated framework for wildfire management that balances model fidelity, computational efficiency, and decision intelligence. Through its modular architecture, stochastic foundations, and fuzzy inference system, DISASTERAWARE establishes a robust, scalable approach to wildfire disaster response. The ongoing work focuses on implementing the full decision-action loop and validating performance through simulated and scenario-based experiments.

# Introduction

Wildfires are among the most destructive and rapidly evolving natural hazards, exacerbated by climate change, land-use transformations, forest fuel accumulation, and increasing anthropogenic pressures. According to global statistics, more than 67 million hectares of forest and wildland areas were burned between 2003 and 2012, leading to devastating ecological losses, substantial carbon emissions, and direct threats to human settlements and economic infrastructures (Doerr & Santín, 2016; San-Miguel-Ayanz, Durrant, & Pivetta, 2018). Contemporary challenges such as prolonged droughts, urban-wildland interface expansion, and extreme weather conditions further elevate both the frequency and severity of fire events.

To counteract these hazards, wildfire management systems must provide:

- early detection,

- accurate modelling of spatio-temporal fire spread, and

- real-time decision support for resource deployment and intervention planning.

However, the inherent complexity of wildfire behavior, driven by non-linear interactions among topography, meteorology, vegetation, and human activity, renders deterministic or rule-based systems insufficient (Finney M. , 2002; Sullivan, 2009). Semi-empirical models such as Rothermel’s surface fire spread model (Rothermel, 1972) offer an operational compromise between computational tractability and predictive performance, and they constitute the foundation of widely used fire behavior tools such as FARSITE (Finney M. , 1998), FlamMap (Finney M. , 2006), and BEHAVE (Andrews, 1986). These models calculate fire spread rate by accounting for fuel characteristics, wind speed, and slope but operate under simplified physical assumptions and lack built-in decision-making capabilities.

Nevertheless, simulation models alone are insufficient for practical decision-making, especially under uncertainty, limited time, and resource constraints. In response, the scientific community has proposed Decision Support Systems (DSS) to bridge the gap between fire dynamics simulation and real-world intervention decisions (Martell, 2015; Calkin, Finney, & Ager, 2011). DSS aims to generate prioritized, feasible, and adaptive recommendations by processing dynamic spatial data, forecasting fire progression, and evaluating multiple intervention strategies. However, existing systems typically suffer from one or more of the limitations listed in Table 1.

Table 1 Gap Analysis

| Problem Area | Limitation in Existing Work | DISASTERAWARE Solution |
| --- | --- | --- |
| Model–Decision Disconnect | Most DSS do not integrate real-time fire spread simulation or consider terrain/resource delays (Calkin, Finney, & Ager, 2011; Noori, 2020) | Fully integrated spatio-temporal grid-based wildfire simulator using the Rothermel model, with dynamic fire progression influencing fuzzy decision logic and resource movement incorporated via Tarr, Vair, Vgnd. |
| Uncertainty Handling | Very few DSS quantify simulation reliability or incorporate confidence in action generation (Thompson & Calkin, 2011; Riley, 2018) | Introduces a Prediction Confidence Score based on the Coefficient of Variation (CV) across key simulation outputs; used to modulate decision aggressiveness and rule activation thresholds. |
| Fuzzy Systems for DSS | Fuzzy logic is used mainly for static fire risk zoning, not dynamic decision-making (Adab, Kanniah, & Solaimani, 2013; Yin, 2018) | Implements a real-time fuzzy inference system triggered at each simulation step, operating over dynamic input variables like fire intensity, suppression success, and confidence. |
| Interpretability & Explainability | Deep models (e.g., ANN, RF) and complex fuzzy models with limited interpretability provide black-box decisions with limited transparency (Jain, 2020) | Employs a transparent rule-based fuzzy system (Mamdani-type), with human-interpretable rules, membership functions, and linguistic variable structure aligned with expert knowledge. |
| Multi-objective Decision Evaluation | Most systems lack explicit cost functions or weighting of conflicting outcomes (Finney M. , 2002; Martell, 2015) | Defines a multi-criteria cost function, aggregating ecological loss, suppression cost, emission, affected population, and fuel residuals. Includes adaptive weights and periodic re-evaluation. |
| Lack of Resource Mobility Models | Few systems include dynamic delays for aerial/ground unit travel and intervention effectiveness (Finney M. , FlamMap Fire Spread Simulator, 2006; Scott & Reinhardt, 2001) | Models’ mobility delay using explicit parameters (e.g., Vair, Vgnd, Tarr), and suppression success as a function of resource type, arrival time, and cell state progression. |

DISASTERAWARE, a modular, adaptive, and fuzzy logic-based DSS architecture designed for wildfire response planning and intervention optimization. The proposed framework is Rothermel-model-driven, grid-based, and supports spatio-temporal simulation with uncertainty propagation and intelligent decision generation. Key contributions of this study include:

- Real-time wildfire simulation integrated with dynamic grid-cell updates and fuel-consumption feedback;

- A fuzzy rule-based inference engine, trained on fire intensity, intervention costs, suppression effect, and confidence metrics;

- Incorporation of Coefficient of Variation (CV) to compute a prediction confidence score for stochastic input variability;

- Design of a multi-objective cost function accounting for ecological, economic, operational, and societal losses;

- Modelling of resource mobility and delayed suppression effectiveness for both aerial and ground assets;

- A fully interpretable decision architecture compliant with operational needs and scenario explainability.

This study positions itself at the intersection of simulation-based forecasting, stochastic modelling, and explainable AI-driven DSS, providing a novel methodology for operational wildfire management under uncertainty.

# System Architecture

Figure 1 Overall System Diagram

The overall system diagram for DISASTERAWARE is illustrated in Figure 1. The system has a Simulation Parameters Database, which stores the necessary initial conditions and data for running wildfire simulations. This database includes detailed parameters, such as fuel types, moisture content, wind speed and direction, slope angles, and other essential attributes defined clearly in Table 6 and Table 7. These parameters serve as initial conditions and are continuously updated by feedback received from the Decision Support System (DSS).

These simulation parameters are passed into the Rothermel Fire Simulation, a core computational module of the system, employing the equations and methodology detailed in Table 4 (Rothermel Model Equations) and Table 5 (Simulation Steps). The Rothermel simulation calculates critical wildfire behavior attributes, including the fire spread rate, fire spread direction, ignition times, and cell states (Unburned, Burning, Burned) on a cell-by-cell basis. It dynamically updates each cell's state and related attributes, such as resource usage and local population impacts, continuously.

The updated results from the Rothermel simulation are passed to the Map Grid Database, a structured storage component holding real-time spatial-temporal information. This database contains cell-specific data such as current ignition states, resource utilization details, and updated population information. It also integrates updates triggered by the DSS interventions, ensuring that all spatial data remains current and comprehensive.

The data from the Map Grid Database feeds into the Grid Map (User Interface). This graphical interface visualizes the ongoing wildfire simulation status, including burned areas, active fire fronts, resources deployed, and population impacts, enabling operational decision-makers to grasp real-time situational awareness effectively. Operators can clearly observe how interventions affect wildfire dynamics and resource utilization in a spatial-temporal context.

The Decision Support System (DSS) receives continuous, real-time updates from the Grid Map Database, reflecting changes in fire behavior, resource status, and affected populations. The DSS evaluates these updates, makes informed intervention decisions (e.g., resource deployment, suppression activities, evacuation orders), and feeds these intervention actions back to both the Rothermel Fire Simulation and the Map Grid Database. The DSS also reads measurement feedback to assess intervention effectiveness and prediction confidence, guiding future decision-making and refining intervention strategies dynamically.

The DSS aims to minimize a clearly defined Cost Function described in Table 9. This cost function aggregates multiple criteria, including total burned area, economic losses, suppression costs, affected population, carbon emissions, residual fuel load, intervention effectiveness, and prediction confidence. By continuously assessing outcomes against this cost function, the DSS optimizes decisions, aiming to minimize overall wildfire impact and costs. The cost function thus forms the central objective metric for assessing the effectiveness and efficiency of the system’s wildfire management strategies.

Overall, the diagram clearly and accurately captures the integrated flow of data and decision-making processes, emphasizing dynamic feedback loops between simulation models, spatial databases, decision-making components, and cost optimization goals. This comprehensive, iterative structure provides a robust foundation for operational and effective wildfire disaster response and management.

# Technical Background

## Review of Wildfire Simulation Models

This report provides an academic review and comparative analysis of the primary wildfire spread simulation models widely utilized in forest fire management and Decision Support Systems (DSS). Each method is described in detail with their inventor, date, relevant literature references, distinctive features, and limitations clearly outlined. The simulation models are described in Table 2and the comparison result is listed in Table 3, relatively.

Table 2 SoA Models and their key features with limitations

| Model | Key Features | Limitations |
| --- | --- | --- |
| ROTHERMEL Fire Spread Model (Rothermel, 1972) | - Combines empirical and physical modelling approaches. - Widely accepted as an international standard. - Efficient computational requirements, allowing rapid simulation. - Forms the foundation for many wildfires simulation software (e.g., BEHAVEPlus (Andrews, 1986), FlamMap (Finney M. , 2006), FARSITE (Finney M. , 1998)). | - Primarily designed for surface fires; limited accuracy in modelling crown fires. - May underestimate or overestimate fire spread in specific scenarios (high wind conditions, heavy fuels). |
| Prometheus (Canadian Forest Fire Behavior Prediction - FBP Model) (Tymstra, Bryce, & Wotton, 2010) | - Empirical model specifically calibrated for Canadian forests and fuels. - Uses locally-specific fuel types, providing high accuracy for Canadian conditions. - Performs effectively in simulating large-scale wildfire spread in boreal ecosystems. | - Requires adaptation for accurate use in other geographic regions. - Region-specific parameters limit broad international applicability. |
| Phoenix RapidFire (Australia) (Tolhurst, Chong, & Duff, 2008) | - Based on McArthur Forest and Grassland Fire Danger Models. - Models’ complex interactions (e.g., wind-slope, self-extinction phenomena, and spotting). - Particularly effective in modelling rapid fire spread and short/long-range ember spotting. - Widely validated within Australian landscapes and vegetation. | - Region-specific; requires significant adaptation outside Australia. - High computational demands for detailed spatial modelling. |
| Minimum Travel Time (MTT) Algorithm - Finney’s Model (Finney M. , 2002) | - Uses a mathematically optimized algorithm (MTT) to rapidly calculate fire spread. - Basis for FlamMap, FSPro, Randig, and FSim simulations. - Enables fast and efficient simulations of thousands of wildfire scenarios. - Extensively validated and widely adopted by land management agencies internationally. | - Simplified representation of complex fire dynamics (spotting, crown fires). - May not fully capture nuanced fire behaviors under extreme conditions. |
| Physics-Based Models (FIRETEC, WFDS) (Linn, Reisner, & Colman, 2002; Mell, Jenkins, & Gould, 2007) | - Fully physics-based, detailed fire modelling (combustion, fluid dynamics, heat transfer). - Provides high-fidelity and highly detailed predictions of fire behavior and impacts. - Especially valuable for research applications, lab-scale validation, and detailed scenario analysis. | - Requires intensive computational resources; difficult to scale for large landscapes. - Not practical for operational, real-time wildfire management due to complexity and high computational load. |

Table 3 Comprehensive Comparative Table of Fire Simulation Models

| Model Name | Year | Inventors/Developer | Validity & Applicability | Accuracy | Computational Load | Real-World Usage |
| --- | --- | --- | --- | --- | --- | --- |
| Rothermel Model | 1972 | Richard C. Rothermel | Global, widely validated | Moderate-High | Low | Very High |
| Prometheus (Canadian FBP) | 1999 | Forestry Canada (1992), Alberta Sustainable Resource Development, Canadian Forest Service | High (Canada, Boreal forests) | High (Canada) | Medium | High (Canada) |
| Phoenix RapidFire | 2008 | Tolhurst et al., University of Melbourne | High (Australia-specific) | Very High | Medium-High | High (Australia) |
| Minimum Travel Time (MTT) | 2002 | Mark A. Finney (FlamMap, FSPro, FSim) | Very high, global | High | Medium | Very High |
| FIRETEC & WFDS | FIRETEC: ~2002 WFDS: 2007 | Linn et al. (FIRETEC), Mell et al. (WFDS) | Scientific validation (global research) | Very High (detailed physical accuracy) | Very High (Complex) | Limited operational use |

While Rothermel-based models remain the most suitable for real-time DSS integration due to their computational simplicity, it is worth noting that advanced models like MTT and FIRETEC offer promising potential for hybrid integration. For example, MTT’s fast-path optimization can enhance the tactical resource allocation layer, while FIRETEC’s physics-based outputs may enrich the observation space of the DSS with higher fidelity intensity profiles or crown fire risks. However, their high computational cost and limited scalability currently restrict real-time applicability. Future work could investigate offline–online hybrid architectures where FIRETEC/MTT run in pre-simulation mode to calibrate fuzzy rule boundaries or initialize decision weights. Considering academic validation, operational usage, practicality, computational load, and general acceptance in wildfire management, the following recommendations emerge clearly from the literature:

For Operational DSS and Real-World Implementation:

  - Rothermel-based models (FARSITE, FlamMap, FSPro, MTT) remain the most suitable due to their empirical validation, computational efficiency, and broad acceptance in operational management.

For Region-specific scenarios (Canada, Australia):

  - Prometheus (Canada), Phoenix RapidFire (Australia) provide greater accuracy due to region-specific fuel models and calibration, recommended if data and conditions match closely.

For Academic Research, Validation, and Detailed Physical Simulations:

  - FIRETEC and WFDS models represent the highest level of detailed accuracy and physical realism, ideal for intensive research purposes, but limited in operational usage due to computational requirements.

Considering comprehensive academic and practical evaluations, the Rothermel-based fire spread simulation methods (FARSITE, FlamMap, FSPro, MTT) are currently identified as the most reliable, widely validated, and operationally efficient models for wildfire spread simulations and DSS implementation. Physics-based models (FIRETEC, WFDS), while offering superior detailed accuracy, are largely limited to academic research due to substantial computational demands.

Therefore, for practical DSS development in forest fire management, the Rothermel-based modelling framework remains the recommended standard in most operational and research-oriented contexts.

## Rothermel Based Wildfire Modelling

The Rothermel model (Rothermel, 1972) is widely used for predicting forest fire spread rates. It is a fundamental component of many fire behavior simulation systems used in fire management. This model predicts the rate of fire spread considering parameters such as wind, slope, fuel type, and fuel moisture. The model details are given in Table 4.

Table 4 Rothermel model equations and explanations

| Fire Spread Rate Equation | R =IR× ξ × 1 + φw+ φsρb× ε × Qig R: Fire spread rate (m/min or m/s) IR: Reaction intensity; energy released per unit area of the flaming front (kW/m²) ξ: Propagating flux ratio; ratio of heat contributing to fire spread (dimensionless) φw: Wind coefficient; the impact of wind on fire spread (dimensionless) φs: Slope coefficient; the impact of terrain slope on fire spread (dimensionless) ρb: Bulk density of fuel bed (kg/m³) ϵ: Effective heating number; fraction of energy heating the fuel (approx. 0.9) Qig: Heat of preignition; energy required to ignite fuel (approx. 2500 kJ/kg) |
| --- | --- |
| Reaction Intensity | IR = Γ × Wn × h × ηM × ηs Γ: Maximum reaction velocity (1/min) Wn: Fuel load; amount of fuel per unit area (kg/m²) h: Heat of combustion of the fuel (kJ/kg) ηM: Moisture damping coefficient (dimensionless) ηM=1-2.59(MfMext)+5.11MfMext2-3.52MfMext3 Mf(%)=0.942×RH0.679+11×eRH-10010+0.18×21.1-T×1-e-0.115×RH10 Mf: Fuel Moisture Content (%) Mext: Extinction Moisture Content (%) (see Table 7) T: Atmospheric temperature (°C) RH: Atmospheric relative humidity (%) ηs : Mineral damping coefficient (dimensionless) |
| Propagating Flux Ratio | ξ =exp0.792 + 0.681σ× β + 0.1192 + 0.2595σ σ: Surface-area-to-volume ratio of fuel particles (1/m) β: Packing ratio of the fuel bed (dimensionless) |
| Wind Coefficient | φw = C × UB × ββopt-E βopt: Optimal packing ratio (dimensionless) U: Wind speed (m/s) B, C, E: Empirical coefficients varying by fuel type |
| Slope Coefficient | φs = 5.275 × β-0.3 × tan θslope2 θslope: Slope angle of terrain (radians) |
| Bulk Density of Fuel | ρb =Wnδ δ: Depth of the fuel bed (m) |
| Effective Heating Number | ε ≈ 0.9 |
| Heat of Preignition | Qig ≈ 2500kJkg |

## Fuzzy Logic in DSS

Fuzzy logic extends binary (true/false) reasoning to degrees of truth, enabling modeling of real-world uncertainty. Since Lotfi Zadeh's foundational work in 1965, it has evolved dramatically, bridging the gap between human qualitative reasoning and algorithmic decision-making (Zadeh, 1975). Recent surveys reaffirm fuzzy logic’s importance in interpretability and hybrid AI systems (Gerdes & Acar, 2024; Varshney & Torra, 2023).

Fuzzy logic enables reasoning with linguistic terms such as "hot", "medium", or "cold", each defined by a membership function that assigns a degree of truth between 0 and 1. Unlike classical Boolean logic that only allows binary outcomes (0 or 1), fuzzy logic can handle partial truths. As a result, it is especially effective for systems that require a human-like reasoning approach (Dubois & Prade, 1980).

Let X be a universe of discourse. A fuzzy set A in X is defined by a membership function:

μA : X→ [0,1]

where μA(x) denotes the degree of membership of element x∈ X in the fuzzy set A.

For example, for a fuzzy set "High Temperature", μHigh(x) = 0.8 implies x belongs to the "High" category with 80% degree of truth.

One of the most commonly used membership function types is the triangular membership function:

μ(x;a,b,c)=0x≤a or x≥cx-ab-aa<x≤bc-xc-bb<x<c

Where: (illustrated in

- a: left vertex (start of slope)

- b: peak point (maximum membership)

- c: right vertex (end of slope)

Figure 2 Example of Triangular Membership Functions

This figure shows three fuzzy sets: “Low,” “Medium,” and “High” defined over the same universe x∈ [0,10]. Their overlap demonstrates smooth transitions, which are core to fuzzy logic’s effectiveness in control and reasoning systems.

A fuzzy inference system (e.g., Mamdani-type) employs rules of the form:

IF x is Low AND y is High THEN z is Medium

- Each linguistic variable is mapped using a fuzzy set.

- Inputs are fuzzified, combined using logical operators (min/max or t-norms), and mapped to output membership functions.

- The output fuzzy set is then "defuzzified" (e.g., using centroid method) to yield a crisp action.

# Wildfire Simulation with DSS Integration

## Algorithmic Flow

To perform a spatial-temporal simulation, the landscape should be divided into grid cells (e.g., 30x30 meters per cell). Each cell will have distinct attributes, including fuel properties, slope, wind, and moisture conditions. The implementation steps are listed in Table 5.

Table 5 Simulation Steps

| Step 1: Initialization (Appendix 8.1.1, 8.1.2) | - Define ignition points and ignition time. - Initialize grid cell attributes: - Fuel model and fuel loading - Slope angle from DEM (Digital Elevation Model) - Wind speed and direction - Fuel moisture content (live and dead fuels) |
| --- | --- |
| Step 2: Fire Spread Rate Calculation (Appendix 8.1.3) | Calculate fire spread rate (R) for each grid cell at every timestep using the Rothermel equation given in Table 4. R =IR× ξ × 1 + φw+ φsρb× ε × Qig |
| Step 3: Determining Fire Spread Direction (Appendix 8.1.3) | Calculate the direction of fire spread combining wind and slope influences: θfire = ww × θwind + ws × θslope θwind: Wind direction radians, θslope: Slope direction (radians) ww and ws: weighting factors depending on wind and slope intensity ws+ww=1 θfire vs θwind and θslope for ws=ww=0.5 is illustrated in Figure 3 |
| Step 4: Calculation of Spread Direction and Spread Time and to Neighboring Cells (Appendix 8.1.3) | Compute time required for fire to spread from one cell to adjacent cells: Tspread = dcellmR × fθfire, θnbr dCell: Side length of the cell (m) fθfire, θneighbor: Directional factor considering alignment between fire spread direction and direction to neighbor cell f(θfire,θnbr)=max[0,cos(θfire-θnbr)] Θfire: Direction of fire spread from the current cell (radians or degrees) θneighbor: Direction from the current cell to the neighboring cell (radians or degree) fθfire,θnbr=max0,cosθfire-θnbr vs θfire,θnbr is illustrated in Figure 4. |
| Step 5: Updating Cell States and Grid map (Appendix 8.1.4) | For each cell, update the ignition time based on minimum arrival times: Ignition Time: Tign=Tcurrent+Tspread Update cell states: Cell State=UnburnedTcurrent<TignBurningTcurrent≥Tign and Wn>Wn, th and IR>IR, thBurnedWn≤Wn,th or IR≤IR,th or Tcurrent≥Tign+TmaxBurn Wn,th=0.1 x Wn,init IR,th=0.1 x IR,init TmaxBurn=180 min Resources: Rused,air=0TopRused,airτdτ Rair,new=Rair,old- Rused,air Rused,ground=0TopRused,groundτdτ Rground,new=Rground,old-Rused,ground Population: @ gridi: Population=Population density-Evacuated Population Pi=Dpop-Pevac,i |
| Step 7: Sim outputs (Appendix 8.1.5, 8.1.6) | See Table 9 |
| Step 8: Simulation Time (Appendix 8.1.5) | Advance the simulation by the defined time increment (e.g., every minute): Recalculate parameters for each cell. Repeat Steps 2-7 for each time increment until the simulation end time is reached. |

Figure 3 θfirevs θwind and θslope for ws=ww=0.5

Figure 4 fθfire,θnbr=max0,cosθfire-θnbr vs θfire,θnbr

## Simulation Input Parameters

The simulation parameters are listed in Table 6 and Table 7. These parameters are initially set and update with respect to the actions taken according to the decisions given in enhanced-DSS.

Table 6 Simulation input parameters by category

| Cat. | Func | Simulation Input Parameter | Symbol | Range Values/Units | Time Dep. | Loc. Dep. |
| --- | --- | --- | --- | --- | --- | --- |
| Fuel Characteristics | Rothermel Model | Fuel Model | - | Grass, Brush, Timber, Slash | No | No |
|  |  | Maximum Reaction Velocity | Γ | 0.01-1.0 1/min | No | Yes |
|  |  | Fuel Load | Wn | 0.1-10 kg/m^2 | No | Yes |
|  |  | Heat of Combustion | h | 16000-25000 kJ/kg | No | Yes |
|  |  | Mineral Damping Coefficient | ηs | 0.0-1.0 | No | Yes |
|  |  | Surface Area-to-Volume Ratio | σ | 500-20000 1/m | No | Yes |
|  |  | Packing Ratio | β | 0.01-0.3 | No | Yes |
|  |  | Optimal Packing Ratio | βopt | 0.05-0.2 | No | No |
|  |  | Fuel Bed Depth | δ | 0.1-2.0 m | No | Yes |
|  |  | Effective Heating Number | ϵ | 0.7-1.0 (typ 0.9) | No | No |
|  |  | Heat of Preignition | Qig | 2000-3000 kJ/kg | No | Yes |
| Meteorology | Fuel Moisture | Atmospheric Temperature | T | -20°C - 50°C | Yes | Yes |
|  |  | Atmospheric Relative Humidity | RH | 0 - 100% | Yes | Yes |
| Wind and Slope Parameters | Rothermel Model | Wind coefficient factor (wind exponent) | B | 0.5-2.0 (empirical) | No | No |
|  |  | Wind coefficient factor | C | 0.001-0.1 (empirical) | No | No |
|  |  | Wind coefficient factor | E | 0.01-2.0 (empirical) | No | No |
|  |  | Wind Speed | U | 0-100 km/h | Yes | Yes |
|  |  | Slope Angle | θ | 0°-60° (degrees or radians) | No | Yes |
|  | Direction | Wind Direction | θwind | 0°-360° | Yes | Yes |
|  |  | Slope Direction | θslope | 0°-360° | No | Yes |
|  |  | Wind Weighting Factor | ww | 0.0-1.0 (sum to 1, typically 0.5-0.9) | Yes | No |
|  |  | Slope Weighting Factor | ws | 0.0-1.0 (sum to 1, typically 0.1-0.5) | No | No |
| Grid Parameters | Spread Time | Cell Size | dcell | 5-100 m (typically 30 m) | No | No |
|  | Fire Spread Direction | Fire spread direction | θfire | 0°-360° (degrees or radians) | Yes | Yes |
|  |  | Neighbor Cell Direction | θnbr | 0°-360° (degrees or radians) | Yes | Yes |
|  | Map | Elevation (DEM) | - | 0-5000 m | No | Yes |
|  | Location | Ignition Point Location | - | Coordinates (X, Y) | No | No |
| Sim Params | Timing | Ignition Time | - | Hour: Minute (e.g., 14:30) | No | No |
|  |  | Simulation Duration | - | minutes/hours/days | No | No |
|  |  | Time Step | - | 1 min-1 hour (typ. 1 min) | No | No |
| Operation | Resource | Aerial Resource Unit Cost | Cunit,aerial | USD/unit resource/hour | No | No |
|  |  | Aerial Resource Amount | Raerial | 0-max resource unit | Yes | Yes |
|  |  | Aerial Resource Reachable Region | XYair | 0-limits of map | No | Yes |
|  |  | Actual Aerial Resource Suppression Effect | Sair,act | 0-total fuel load (kg) | Yes | Yes |
|  |  | Aerial Resource Suppression Effect Rate | Sair,rate | 0-100 kW/(m2 min) typ. 10 kW/m²/min) | No | Yes |
|  |  | Aerial Resource Speed | Vaerial | 50-150 km/h | No | No |
|  |  | Ground Resource Unit Cost | Cunit,gnd | USD/unit resource/hour | No | No |
|  |  | Ground Resource Amount | Rgnd | 0-max resource unit | Yes | Yes |
|  |  | Ground Resource Reachable Region | XYgnd | 0-limits of map | No | Yes |
|  |  | Actual Ground Resource Suppression Effect | Sgnd,act | 0-total fuel load (kg) | Yes | Yes |
|  |  | Ground Resource Suppression Effect Rate | Sgnd,rate | 0-50 kW/m²/min typ. 5 kW/m²/min | No | Yes |
|  |  | Ground Resource Speed | Vgnd | 60-250 km/h | No | No |
|  | Population | Cell Population density | Dpop | 0-0.3 /m2 | Yes | Yes |
|  | Economic | Economic value per unit area | Cval | 0-unlimited (USD/m2 ) | No | Yes |
|  | Environment | Emission Factor | Efactor | 0-50 (kg emissions/kg fuel) | No | Yes |

Table 7 lists the value look up table for B, C , E, Mext, ηs  values used in Rothermel equations and the simulation parameters described in Table 4 and Table 6. The values depend on the studies available in the literature (Anderson, 1982; Scott & Burgan, 2005; Rothermel, 1972).

Table 7 Fuel-Dependent Parameter Lookup Table

|  | Fuel Model | Description | B | C | E | Mx,ext | ηs | Efactor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grass | Model 1 | Short grass | 1.03 | 0.045 | 0.3 | 12% | 0.417 | 1.60 |
|  | Model 2 | Timber with grass and understory | 1.05 | 0.030 | 0.4 | 15% | 0.400 | 1.65 |
|  | Model 3 | Tall grass | 1.04 | 0.035 | 0.4 | 25% | 0.380 | 1.70 |
| Brush | Model 4 | Light brush (chaparral) | 1.20 | 0.015 | 0.3 | 20% | 0.368 | 1.80 |
|  | Model 5 | Medium-density brush | 1.25 | 0.020 | 0.35 | 20% | 0.350 | 1.90 |
|  | Model 6 | Dormant brush | 1.30 | 0.025 | 0.35 | 25% | 0.330 | 2.00 |
|  | Model 7 | Southern rough heavy brush | 1.35 | 0.030 | 0.4 | 40% | 0.320 | 2.10 |
| Timber | Model 8 | Closed timber with short litter | 1.40 | 0.008 | 0.25 | 30% | 0.300 | 1.85 |
|  | Model 9 | Hardwood litter | 1.45 | 0.010 | 0.3 | 25% | 0.290 | 1.80 |
|  | Model 10 | Timber litter with understory | 1.50 | 0.012 | 0.3 | 25% | 0.280 | 1.90 |
| Slash | Model 11 | Light logging slash | 1.55 | 0.005 | 0.2 | 15% | 0.250 | 1.95 |
|  | Model 12 | Medium logging slash | 1.60 | 0.007 | 0.25 | 20% | 0.230 | 2.00 |
|  | Model 13 | Heavy logging slash | 1.65 | 0.010 | 0.3 | 25% | 0.210 | 2.10 |

## Uncertainty of Simulation Input Parameters

In real-world wildfire scenarios, uncertainty is inevitable due to environmental variability, data limitations, and system complexity. To enhance the robustness and reliability of the Decision Support System (DSS), stochastic modelling and uncertainty quantification must be integrated into both simulation and decision processes. This section presents the key sources of uncertainty, mathematical formulations of stochastic parameters, confidence evaluation via Coefficient of Variation (CV), and the impacts of uncertainty on DSS performance. The sources of uncertainty are listed in Table 8.

Table 8 Categorization of Uncertainty Sources

| Parameter | Symbol | Distribution Type | Distribution Parameters | Justification |
| --- | --- | --- | --- | --- |
| Wind speed | U | Normal | μ=40kmh, σ=10kmh (range: 0-100 km/h) | U fluctuates temporally and spatially; modeled as Normal due to central tendency in short-term forecasts. Affects φw, θfire, and R. |
| Wind direction | θwind | Uniform | min=0°, max=360° | Direction is inherently circular and unbounded within [0°, 360°]; impacts θfire and directional factor in Tspread. |
| Relative humidity | RH | Truncated Normal | μ=50%, σ=15%, bounds: 10-90% | RH varies naturally within a bounded interval. Strongly influences M → ηM → IR → fire spread rate R. |
| Packing Ratio | T | Normal | μ=25°C, σ=10°C (range: -20°C to 50°C) | Temperature follows a Gaussian profile over time; affects M and thus ηM and ignition energy Qig. |
| Fuel Bed Depth | θslope | Uniform | min=0°, max=360° | May vary due to terrain resolution; impacts slope coefficient φs and fire direction θfire. |
| Atmospheric Temperature | Wn | Triangular | min=0.1, mode=2.5, max=10 kg/m² | Dependent on vegetation and aging. Strongly impacts IR and burn duration Tburn. |
| Atmospheric Relative Humidity | δ | Uniform | min=0.1, max=2.0 m | Sampled from terrain vegetation maps; affects bulk density ρb and fire energy per volume. |
| Wind Speed | σ | Uniform | min=500, max=200001m | Depends on fuel type (grass, brush, timber); impacts ξ and fire intensity propagation. |
| Wind Direction | β | Uniform | min=0.01, max=0.3 | Empirically estimated; affects ξ, φs, and φw calculation. Highly uncertain in field. |
| Slope Direction | ηs | Uniform | min=0.0, max=1.0 | Depends on soil and ash content; empirical estimate. Strong effect on IR reduction. |
| Aerial Resource Suppression Effect Rate | Sair | Normal | μ=10, σ=2kWm2min (range: 0-100) | Performance varies by drop accuracy and wind; affects suppression time and intervention effectiveness. |
| Aerial Resource Speed | Sgnd | Uniform | min=0, max=50kWm2min | Varies by equipment type and terrain; critical in computing IR reduction. |
| Ground Resource Suppression Effect Rate | Vair | Normal | μ=120kmh, σ=15 (range: 50-150kmh) | Speed depends on flight dynamics and meteorological factors. Affects arrival delay Tarr . |
| Ground Resource Speed | Vgnd | Normal | μ=90kmh, σ=20 (range: 60-250kmh) | Road accessibility and vehicle type cause uncertainty. Influences deployment timing. |

In order to evaluate the reliability of the simulation outcomes under stochastic variability, a Prediction Confidence metric is introduced. This metric quantifies how much the simulation outputs deviate across multiple stochastic runs, based on the Coefficient of Variation (CV). A lower CV indicates a more stable outcome and thus a higher prediction confidence.

For a given output metric X, its CV is calculated as:

CVX=σXμX

Where:

- μX: mean of the output metric across N stochastic simulations

- σX: standard deviation of the same output

This metric is dimensionless and enables comparing variability across quantities with different units and magnitudes.

To account for multiple relevant metrics in the simulation (e.g., burned area, cost, fuel load), an overall uncertainty level is calculated as the sum of individual CVs:

CVoverall=meanσAburntμAburnt,σCecoμCeco,σCsuppμCsupp,σWnμWn,σCemmμCemm,σPaffμPaff

This aggregate CV reflects the joint variability of all cost-relevant simulation outputs. These outputs are already defined in Table 9 (Simulation Output) and are integrated into the cost function in Section 0. The overall prediction confidence Pconf is computed as:

Pconf=100×1-CVoverall

This transformation yields a score between 0 and 100:

High Confidence (80-100%): Simulation results are consistent, reliable, and have low uncertainty.

Moderate Confidence (60-80%): Results are acceptable but somewhat uncertain; additional data or repetitions may be required.

Low Confidence (<60%): Results are highly uncertain; more data, simulation repetitions, or improvements are necessary.

Use of Prediction Confidence in DSS is as follows:

Rule Strength Adjustment: Confidence can be used as a meta-variable in fuzzy inference to modulate the aggressiveness of actions.

Intervention Thresholding: DSS may delay or moderate actions if confidence is too low.

Cost Function Penalty: The term 1-Pconf is penalized in the DSS cost function in Section 0, to avoid decisions based on unreliable forecasts.

## Simulation Outputs

The simulation output parameters are listed in Table 9. The output parameters are initially set and these parameters change with respect to the actions taken according to the decisions given in enhanced-DSS.

Table 9 Simulation output

| Output Parameter | Unit | Is it sim param? | Is it cost function param? | Calculation Formula or Method |
| --- | --- | --- | --- | --- |
| Burn duration | Hr | Yes | No | Tburn=Wn×h×δIR-Srate Srate= Sgnd,rate+ Sair,rate |
| Operation Duration (Resource Mobility and Timing) | Hr | Yes | No | Top=Tarr+Tsupp+Tmopup Tarr=(iRes-iBurning)Vres iRes:location of the resource iBurning:location of the burning cell Vres:Speed of the resource it is either Vaerial or Vgnd Tsupp= IR-IR,thSres,act=IR×0.9Sres,act , IR,th=IR×%10 Tmopup= Tmopup,base×Wn×1(Rgnd+Rair) If Tarr>Tburn then intervention is meaningless. |
| Aerial Utilized Resource Amount | lt | Yes | No | Rair,used=0TopRair,usedτdτ Rair=Rair,init- Rair,used |
| Ground Utilized Resource Amount | lt | Yes | No | Rgnd,used=0TopRused,gndτdτ Rgnd=Rgnd,init-Rgnd,used |
| Residual Fuel Load | kg/m² | Yes | Yes | ResidualFuelLoad=InitialFuelLoad-ConsumedFuel ConsumedFuel≈IR×TBurnh Wn=Wn,init-IR×TBurn h |
| Carbon Emission | kg/m² | No | Yes | Wcons=IR×TBurn h (Consumed Fuel) Carbem=Wcons×Efactor |
| Total Suppression Cost | USD | No | Yes | Csupp= Cunit,aerial× Raerial ×Top+ Cunit,gnd× Rgnd ×Top |
| Total economic loss | USD | No | Yes | Ceco=iCval,i |
| Total Burned Area | m² | No | Yes | Aburnt=Nburnt×dCell2 Nburnt: Number of burned cells |
| Affected Population | persons | No | Yes | Arisk=Nrisk×dcell2 Arisk: Area at risk m2 Nrisk: Number of risk cells Prisk=Arisk×Dpop Aevac=Nevac×dcell2 Nburnt: Number of evacuated cells Pevac=Aevac×Dpop Paff= Prisk- Pevac Dpop: Population density (persons/m², external data) |
| Intervention Effectiveness | % (0-100) | No | Yes | IE=wIE,AburntIEAburnt+wIE,ecoIEeco+wIE,suppIEsupp+wIE,affPopIEaffPop+wIE,WnIEWn+wIE,CemmIECemm i=1NwIE,i=1 IEX=Xbase,no_intervention-XinterventionXbase,no_intervention×100 Parameter; Typ. initial value. w1; Burned Area; 0.25. w2; Economic Loss; 0.20. w3; Suppression Cost; 0.15. w4; Affected Population; 0.20. w5; Emissions; 0.10. w6; Residual Fuel Load; 0.10. (Ref to Table 10) |
| Prediction Confidence | % (0-100) | No | Yes | CVoverall=meanσAburntμAburnt,σCecoμCeco,σCsuppμCsupp,σWnμWn,σCemmμCemm,σPaffμPaff Pconf=100×1-CVoverall CV: Coefficient of Variation High Confidence (80-100%): Simulation results are consistent, reliable, and have low uncertainty. Moderate Confidence (60-80%): Results are acceptable but somewhat uncertain; additional data or repetitions may be required. Low Confidence (<60%): Results are highly uncertain; more data, simulation repetitions, or improvements are necessary. |

## Cost Function and Optimization Objective

The objective of enhanced DSS is to minimize the cost function derived from Table 9 (Simulation Outputs):

Jtotal=w1⋅BurnedArea+w2⋅EconomicLoss+w3⋅SuppressionCost

+w4⋅AffectedPopulation+w5⋅CarbonEmissions+w6⋅ResidualFuelLoad

-w7⋅InterventionEffectiveness+w8⋅1-PredictionConfidence

minJ= w1⋅Aburnt+w2⋅Ceco+w3⋅Csupp+w4⋅Paff+w5⋅Carbem+w6⋅Wn-w7⋅IE+w8⋅1-Pconf

Subject to:

Rair + Rgnd ≤ Rtotalavailable

Sair≤ Sairmax,   Sgnd≤ Sgndmax

xr, yr∈ ReachableZone

Δt≤dRmax

Tintervention≥dr→cellVr

Wx, y, t>0 ⇒ R(x, y, t) > 0

i=18wi= 1,  wi∈ [0, 1]

Table 10 Weights of parameters for Cost Function

| Weight | Parameter | Typical initial value | Reasoning for the initial values |
| --- | --- | --- | --- |
| w1 | Burned Area | 0.25 | Finney et al. (2011) emphasize the critical role of burned area in managing large wildfires, demonstrating its direct impact on containment success. Scott (2006) highlights burned area as a primary indicator influencing firefighting strategies and resource allocation. Given the fundamental role identified in these studies, burned area is assigned a relatively high initial weight of 0.25. |
| w2 | Economic Loss | 0.20 | Calkin et al. (2010) discuss economic damages caused by wildfires, identifying economic loss as a significant consequence of wildfire events. Thompson et al. (2013) reinforce that economic factors critically influence wildfire management budgeting and resource allocation. Based on these insights, a substantial initial weight (0.20) is justified to reflect economic importance. |
| w3 | Suppression Cost | 0.15 | Calkin et al. (2011) emphasize the importance of suppression costs as significant budgetary factors impacting wildfire management decisions. Rideout & Ziesler (2008) illustrate that high suppression costs are often critical constraints in operational decision-making. This rationale supports assigning a moderately high initial weight (0.15) to suppression cost. |
| w4 | Affected Population | 0.20 | Venn & Calkin (2011) underline that protecting human life is among the highest priorities in wildfire management, highlighting the significance of the affected population. Similarly, Thompson et al. (2015) emphasize community safety and human impact as paramount in wildfire response planning. Given these findings, an initial weight of 0.20 reflects the critical importance of human safety. |
| w5 | Emissions | 0.05 | North et al. (2015)discuss how emissions influence wildfire management indirectly, and Wiedinmyer & Neff (2007) quantify emissions' contribution to environmental impact and climate change. Although significant, emissions typically have lower immediate priority compared to human and economic factors, leading to a smaller initial weight of 0.05. |
| w6 | Residual Fuel Load | 0.05 | Agee & Skinner (2005)highlight residual fuel load as crucial for reducing future fire risks. Scott & Burgan (2005) further support that remaining fuels critically impact subsequent fire regimes and management efforts. Given its indirect yet critical influence on future wildfire risks, a moderate initial weight of 0.05 is assigned. |
| w7 | Intervention Effectiveness | 0.07 | Calkin et al. (2014)demonstrate the significance of effective intervention strategies in reducing overall wildfire damage. Martell (2015)also reviews how critical effectiveness of interventions directly influences wildfire management success. Thus, intervention effectiveness, as a positive performance indicator, receives a moderate initial weight (0.07) for positive reinforcement. |
| w8 | Prediction Confidence | 0.03 | Thompson & Calkin (2011)and Riley et al. (2018) discuss the importance of uncertainty and confidence in predictions affecting the reliability of wildfire management decisions. Although important, prediction confidence typically serves a supportive role and therefore receives a lower initial weight of 0.03. |

The weights will be updated with the following equation.

winew = wiold +α⋅ Jold- JnewJoldwiold

α:Learning rate (~0.05)

The weight update formula enables adaptive adjustment of objective weights based on observed performance. When the updated decision results in a lower cost Jnew<Jold the system reinforces the weight wi that contributed to improvement. Conversely, underperformance leads to weight reduction. This mechanism allows the DSS to progressively learn which objectives are more critical under different fire scenarios and align its decision strategy accordingly. The learning rate α controls the sensitivity of updates and ensures stability during adaptation.

# DSS Decision-Making Framework

The Decision Support System (DSS) module of DISASTERAWARE constitutes the intelligent core of the wildfire management architecture. Operating atop real-time simulation outputs, it processes dynamic environmental observations, evaluates multiple intervention alternatives, and generates actionable decisions in a transparent and interpretable manner. Unlike static or rule-based systems, this framework adopts a fuzzy logic-based inference engine capable of reasoning under uncertainty, encoding expert knowledge through linguistic variables and membership functions. By integrating stochastic simulation outputs—such as fire intensity, suppression delay, resource availability, and prediction confidence—into its observation space, the DSS continuously adapts its reasoning process. It selects the most appropriate intervention strategy from a predefined action space to minimize a multi-objective cost function. The DSS decision-making cycle is modularly structured into observation encoding, rule-based inference, fuzzy evaluation, defuzzification, and output integration. This section presents the architecture and mechanics of the DSS, detailing how spatial-temporal simulation data are transformed into actionable, cost-aware, and uncertainty-adaptive wildfire intervention decisions.

## Observation Space Definition

Let the observation vector at time t be denoted by:

ot=[ot1,ot2,…,otn]

where each element oti represents a measurable or computable feature of the fire environment, simulation state, or resource condition observed at time t.

Table 11 Observation Space Parameter List

| Parameter | Justification |
| --- | --- |
| R | Primary fire spread rate; directly observed in simulation |
| IR | Reaction intensity is calculated and observable per cell |
| Wn | Observed fuel loading in each cell |
| Tign | Ignition time per cell; stored and updated dynamically |
| Tcurrent | Current simulation time; always known |
| Tspread | Calculated for fire propagation, treated as observable |
| θfire | Fire direction vector; cell-specific |
| θneighbor | Direction to neighbor; known from grid layout |
| fθfire, θneighbor | Directional factor used in spread calc. |
| Cell state | Simulation state: unburned, burning, burned |
| T | Ambient temperature is environment input |
| RH | Relative humidity; part of moisture computation |
| U | Wind speed; time and location dependent |
| θwind | Direction of wind; weather input |
| θslope | Slope direction; from DEM |
| XYair | Reachable zone for aerial units |
| XYgnd | Reachable zone for ground units |
| Vair | Speed of aerial resource |
| Vgnd | Speed of ground resource |
| Sair,act | Actual aerial suppression effect |
| Sair,rate | Rate of aerial suppression |
| Sgnd,act | Actual ground suppression |
| Sgnd,rate | Rate of ground suppression |
| Rair | Remaining aerial resources |
| Rgnd | Remaining ground resources |
| Dpop | Population density in cell |
| Pconf | Prediction confidence metric |
| Tburn | Cell-level burn duration |
| Top | Operation duration (intervention total time) |
| Rair,used | Amount of aerial resource used |
| Rgnd,used | Amount of ground resource used |
| Wres | Residual fuel |
| Paff | Risk per cell; inferred from population map |
| IE | Intervention effectiveness; computed feedback |

The observation space of the DISASTERAWARE Decision Support System comprises a structured set of variables that encapsulate the real-time state of wildfire dynamics, resource availability, and environmental conditions. Table 11 lists the full set of parameters that constitute the observation vector at each decision epoch. These variables include both directly measurable simulation outputs—such as fire spread rate (RRR), reaction intensity (IR), fuel loading (Wn), ignition time (Tign), and directional alignment factors f(θfire,θneighbor)—as well as operational and environmental indicators such as temperature (T), relative humidity (RH), wind direction and speed (θwind, U), and slope characteristics.

In addition, resource-related variables (e.g., Rair,Rgnd, suppression effectiveness rates, speeds, and utilized amounts) are integrated into the observation space to reflect the evolving capabilities of aerial and ground units. The framework also incorporates higher-level strategic metrics such as population density (Dpop), affected population risk estimates, intervention effectiveness (IE), and prediction confidence (Pconf)—the latter of which modulates the aggressiveness of fuzzy rule application. All of these parameters are locally or globally observable by the DSS at runtime and form the basis of its inference and decision generation processes.

This comprehensive observation space ensures that the DSS operates with full contextual awareness of both the physical wildfire behavior and the operational constraints, thereby enabling adaptive and scenario-specific decision-making.

## Action Set and Decision Categories

The DISASTERAWARE Decision Support System (DSS) operates over a structured action space A, consisting of all feasible intervention strategies that can be applied in response to dynamic fire behavior and resource availability. Each decision at time step t involves selecting one action  at∈A, guided by fuzzy rule evaluations and cost function objectives. The action space is modular, decomposed into three orthogonal decision dimensions: intervention type, execution timing, and intervention scale. This modularity ensures flexibility, interpretability, and adaptability to real-time conditions with uncertainty.

Each action is defined as a triplet:

at=Interventiont,Delayt,Scalet

Where:

- Interventiont∈ Table 12

- Delayt∈ Table 13

- Scalet∈ Table 14

The complete fuzzy action space is defined as:

A=Intervention,Delay,Scale Intervention∈I,  Delay∈D,  Scale∈S

where:

- I= Intervention Fuzzy Term in Table 12

- D= Timing Fuzzy Term in Table 13

- S= Scale Fuzzy Term in Table 14

Each fuzzy rule in the system maps specific conditions in the observation space to one of these triplet outputs, for example:

IF FireIntensity is High AND Confidence is High AND Delay is Low THEN Action is (Aerial, Immediate, Full)

Table 12 Intervention Type Categories

| Action Category | Use Cases | Intervention Fuzzy Term (Interventiont) |
| --- | --- | --- |
| Nonaction | Low fire, high uncertainty, or limited resources | Do_nothing |
| Direct Suppression | Rapid cooling of fire front in remote forested areas | Aerial_Water_Drop |
| Direct Suppression | Slowing fire spread across pre-identified containment lines | Aerial_Retardant_Drop |
| Direct Suppression | Suppressing fire in areas with dense fuel and high heat | Aerial_Foam_Drop |
| Direct Suppression | Precision strikes on small, high-risk ignition zones | Drone_Based_Targeted_Suppression |
| Direct Suppression | Manual suppression at flame edge in accessible terrain | Ground_Crew_Direct_Attack |
| Direct Suppression | Water deployment via mobile fire trucks on road-accessible zones | Ground_Engine_Suppression |
| Direct Suppression | Logistical support to maintain continuous ground operations | Ground_Water_Supply_Deployment |
| Direct Suppression | Bulldozers or mechanical tools to create firebreaks | Ground_Equipment_Suppression |
| Direct Suppression | Synchronized resource usage for fast knockdown | Combined_Ground_Aerial_Attack_Simultaneous |
| Direct Suppression | Sequential engagement to maintain pressure over time | Combined_Ground_Aerial_Staggered_ |
| Awareness & Monitoring | Refinement of spread models using real-time fire perimeter data | Fire_Behavior_Prediction_Update |
| Awareness & Monitoring | Data acquisition to calibrate fire intensity and visibility impact | Environmental_Sensing_Meteorological_air_quality_smoke_plume |
| Evacuation & Civil Defense | Immediate warning to populations in high-risk cells | Evacuation_Alert_SMS_Emergency_Loudspeaker_Siren_Trigger |
| Evacuation & Civil Defense | Deployment of buses or vehicles for active zone evacuations | Civilian_Transport_Mobilization |
| Preventive | Pre-fire mitigation in buffer zones to slow spread | Fuel_Removal_Backburn_Fuel_Break_Vegetation_Clearing |
| Logistics & Support | Scaling aerial response to new or growing ignition sites | Deploy_Additional_Aerial_Unit |
| Logistics & Support | Reinforcement of crew numbers at priority suppression fronts | Deploy_Additional_Ground_Unit |
| Logistics & Support | Shifting units based on projected fire spread trajectories | Relocate_Resource_to_Strategic_Zone |
| Communication | Restoring radio/network coverage in comms-denied zones | Deploy_Drone_Based_Communication_Relay |
| Communication | Fallback data connectivity during local network failures | Activate_Satellite_Backup_Link |
| DSS Strategic | Cease action due to ineffectiveness, risk, or updated intel | Abort_Current_Intervention |
| DSS Strategic | Postpone judgment until confidence or observation improves | Delay_Decision |
| DSS Strategic | Run 'what-if' modeling for different intervention strategies | Simulate_Alternate_Scenario |
| DSS Strategic | Adjust strategy focus (e.g., cost vs. lives vs. emission) | Re_evaluate_Cost_Function_Weights |
| DSS Strategic | Modify sensitivity of DSS to simulation uncertainty | Update_Prediction_Confidence_Threshold |

Table 13 Timing Options

| Timing Label | Timing Description | Timing Approx. Delay | Timing Decision Logic | Timing Fuzzy Term Delayt |
| --- | --- | --- | --- | --- |
| Immediate | Action executed at time t | 0 min | Use when fire spread rate is high and resource is ready | Immediate |
| Barely Delayed | Minor safety buffer before execution | ~1 min | Used when fire is intense but minor hesitation exists | Barely_Delayed |
| Slightly Delayed | Minor delay due to confidence or priority | ~2 min | Confidence is moderate, or lower priority cells exist | Slightly_Delayed |
| Moderately Delayed | Moderate deferral | ~5 min | Risk level medium, or distance-related delay | Moderately_Delayed |
| Strongly Delayed | Delay justified by uncertainty | ~10 min | Confidence low, or resource dispatch is uncertain | Strongly_Delayed |
| Critically Delayed | Delay only if no option | ~15 min | Waiting for availability or better conditions | Critically_Delayed |
| Very Strongly Delayed | Extended postponement | >15 min | Severe uncertainty or waiting for resource reallocation | Very_Strongly_Delayed |
| Too Late | Ineffective delay | > T_burn | Resource would arrive after burn completes; ineffective | Too_Late |
| Abort | Cancel action | ∞ | Fire unreachable, or will self-extinguish | Abort |

Table 14 Scale Options

| Scale Label | Resource Allocation | Resource Range (%) | Scale Description | Scale Fuzzy Term Scalet |
| --- | --- | --- | --- | --- |
| Micro | Very low | 5-15% | Probe action, exploratory drop | Very_Low |
| Low | Low suppression | 20-35% | Spot fires, low intensity | Low |
| Medium | Moderate effort | 40-60% | Growing or localized fire | Medium |
| High | Aggressive action | 65-85% | Urban interface, high IR | High |
| Full | All available | 90-100% | Critical zones, high priority | Maximum |

Action feasibility conditions:

Before an action is executed, the following conditions must be verified:

- Reachability:

Check if the target cell is within resource's operational range

x,y∈XYres

(Cell must be within aerial or ground reach)

- Delay check: (Timing Validity)

Confirm if resource can arrive before cell is fully burned

Tarr<Tburn

- Resource sufficiency:

Validate if available resources meet the action's required intensity

Rair+ Rgnd≥ Required for Scale

If any of these conditions fail, the DSS may switch to a “Delay” or “Abort” class action automatically.

## Fuzzy Logic Architecture

Rule-Based Inference System

Membership Function Design

Inference Engine and Rule Evaluation

Defuzzification and Output Decisions

## Rule Set Examples

# Conclusion

# Appendices

## Appendix A: Fully Detailed Pseudocode for Spatial-Temporal Wildfire Simulation

### Initialization Phase

# 1. LOAD DATA LAYERS

fuel_layer = Load_GIS_Layer("Fuel_Model")

dem_layer = Load_GIS_Layer("DEM")

wind_layer = Load_Meteorology("Wind")

moisture_layer = Load_Meteorology("Moisture")

# 2. INITIALIZE GRID

for cell in GridMap:

cell.fuel_model = fuel_layer[cell.id]

cell.fuel_loading = lookup_fuel_loading(cell.fuel_model)

cell.heat_of_combustion = lookup_heat_combustion(cell.fuel_model)

cell.mineral_damping = lookup_eta_s(cell.fuel_model)

cell.moisture_content = compute_moisture_content(cell.location)

cell.eta_M = compute_eta_M(cell.moisture_content, extinction_moisture(cell.fuel_model))

cell.wind_speed = wind_layer[cell.id].speed

cell.wind_dir = wind_layer[cell.id].direction

cell.slope_angle = slope_from_DEM(dem_layer, cell)

cell.slope_dir = slope_direction(dem_layer, cell)

cell.packing_ratio = lookup_beta(cell.fuel_model)

cell.surface_area_to_volume = lookup_sigma(cell.fuel_model)

cell.opt_packing_ratio = lookup_beta_opt(cell.fuel_model)

cell.depth = lookup_depth(cell.fuel_model)

cell.bulk_density = cell.fuel_loading / cell.depth

cell.heat_preignition = lookup_Q_ig()

cell.effective_heating = 0.9  # Typ. value

cell.ignition_time = ∞

cell.state = "unburned"

### Ignition Setup

# Define ignition coordinates manually or through user interface

for ignition_point in ignition_coordinates:

cell = GridMap[ignition_point]

cell.ignition_time = 0

cell.state = "burning"

### Main Simulation Loop

time = 0

time_step = 1  # minute

end_time = TOTAL_SIM_DURATION

while time < end_time:

# 1. UPDATE BURNING CELLS

for cell in GridMap:

if cell.state == "burning":

# 2. Calculate reaction intensity (I_R)

I_R = cell.max_reaction_velocity * cell.fuel_loading * cell.heat_of_combustion * cell.eta_M * cell.mineral_damping

# 3. Calculate propagating flux ratio (ξ)

ξ = exp(((0.792 + 0.681 * sqrt(cell.surface_area_to_volume)) * (cell.packing_ratio + 0.1)) /

(192 + 0.2595 * cell.surface_area_to_volume))

# 4. Calculate wind & slope coefficients

φ_w = C * cell.wind_speed ** B * (cell.packing_ratio / cell.opt_packing_ratio)

φ_s = 5.275 * cell.packing_ratio ** (-0.3) * tan(cell.slope_angle)

# 5. Calculate fire spread rate (R) in m/min

R = (I_R * ξ * (1 + φ_w + φ_s)) / (cell.bulk_density * cell.effective_heating * cell.heat_preignition)

# 6. Calculate fire spread direction (θ_fire)

θ_fire = w_wind * cell.wind_dir + w_slope * cell.slope_dir

# 7. Spread to neighbors

for neighbor in get_adjacent_cells(cell):

θ_neighbor = compute_direction_to(cell, neighbor)

directional_factor = max(0, cos(θ_fire - θ_neighbor))

T_spread = cell_size / (R * directional_factor)

# Check ignition condition

if neighbor.state == "unburned" and time + T_spread < neighbor.ignition_time:

neighbor.ignition_time = time + T_spread

neighbor.state = "burning"

neighbor.reaction_intensity = I_R

neighbor.fuel_loading = cell.fuel_loading  # propagated

update_simulation_outputs(neighbor, time + T_spread)

### Update Cell States & Resources

for cell in GridMap:

# Update State

if time >= cell.ignition_time and cell.state == "burning":

if (cell.fuel_loading <= 0.1 * init_fuel(cell)) or \

(cell.reaction_intensity <= 0.1 * init_intensity(cell)) or \

(time >= cell.ignition_time + T_burn):  # burn duration = 180 min

cell.state = "burned"

# Resource Depletion & Population

if cell.state == "burning":

cell.fuel_loading -= compute_consumed_fuel(cell, time_step)

update_resources(cell, time_step)

update_population(cell, time_step)

### Output & Advance Time

# Update simulation logs, raster export, stats

save_current_state(time)

time += time_step

### Cost Function Evaluation (Periodically)

if time % EVALUATION_PERIOD == 0:

J = w1 * BurnedArea + w2 * EconomicLoss + w3 * SuppressionCost + w4 * AffectedPopulation \

+ w5 * CarbonEmissions + w6 * ResidualFuel - w7 * InterventionEffectiveness + w8 * (1 - PredictionConfidence)

# References

Adab, H., Kanniah, K., & Solaimani, K. (2013). Modeling forest fire risk in the northeast of Iran using remote sensing and GIS techniques. Natural Hazards.

Agee, J., & Skinner, C. (2005). Basic principles of forest fuel reduction treatments. Forest Ecology and Management.

Anderson, H. (1982). Aids to Determining Fuel Models for Estimating Fire Behavior. USDA Forest Service INT-122.

Andrews, P. (1986). BEHAVE: Fire Behavior Prediction and Fuel Modeling System. USDA Forest Service, General Technical Report INT-194.

Calkin, D., Cohen, J., Finney, M., & Thompson, M. (2014). How risk management can prevent future wildfire disasters in the wildland-urban interface. Proceedings of the National Academy of Sciences.

Calkin, D., Finney, M., & Ager, A. (2011). Toward a risk framework for wildfire decision support. Forest Policy and Economics.

Calkin, D., Thompson, M., & Finney, M. (2010). Negative consequences of positive feedbacks in US wildfire management. Forest Ecosystems.

Doerr, S., & Santín, C. (2016). Global trends in wildfire and its impacts: perceptions versus realities in a changing world. Philosophical Transactions of the Royal Society B.

Dubois, D., & Prade, H. (1980). Fuzzy Sets and Systems: Theory and Applications. Academic Press.

Finney, M. (1998). FARSITE: Fire Area Simulator-model development and evaluation. USDA Forest Service, Rocky Mountain Research Station.

Finney, M. (2002). Fire growth using minimum travel time methods. Canadian Journal of Forest Research, 32, 1420-1424.

Finney, M. (2006). FlamMap Fire Spread Simulator. USDA Forest Service, Rocky Mountain Research Station.

Finney, M., Grenfell, I., & McHugh, C. (2011). Modeling containment of large wildfires using generalized linear mixed-model analysis. Forest Science.

Gerdes, W., & Acar, E. (2024). Integrating fuzzy logic into deep symbolic regression: Improving interpretability and performance. Retrieved from https://arxiv.org/abs/2411.00431

Jain, P. (2020). A review of machine learning applications in wildfire science. International Journal of Wildland Fire.

Linn, R., Reisner, J., & Colman, J. (2002). Studying wildfire behavior using FIRETEC. International Journal of Wildland Fire.

Martell, D. (2015). A review of recent forest and wildland fire DSS research. Current Forestry Reports.

Mell, W., Jenkins, M., & Gould, J. (2007). A physics-based approach to modeling grassland fires. International Journal of Wildland Fire.

Noori, H. (2020). Probabilistic modeling of wildfire risk. Journal of Environmental Management.

North, M., Collins, B., & Stephens, S. (2015). Using fire to increase the scale, benefits, and future maintenance of fuels treatments. Journal of Forestry.

Rideout, D., & Ziesler, P. (2008). Three Great Myths of Wildland Fire Management. Wildfire.

Riley, K. (2018). Simulating wildfire decisions using spatial risk. Journal of Forestry.

Rothermel, R. (1972). A mathematical model for predicting fire spread in wildland fuels. USDA Forest Service Research Paper INT-115.

San-Miguel-Ayanz, J., Durrant, T., & Pivetta, E. (2018). Wildfires in Europe: data, assessment and analytical insights. Publications Office of the European Union.

Scott, J. (2006). Comparison of crown fire modeling systems used in three fire management applications. USDA Forest Service RMRS.

Scott, J., & Burgan, R. (2005). Standard fire behavior fuel models. USDA Forest Service RMRS.

Scott, J., & Reinhardt, E. (2001). Assessing crown fire potential. USDA Forest Service.

Sullivan, A. (2009). Wildland surface fire spread modeling, Part 1 & 2. International Journal of Wildland Fire.

Thompson, M., & Calkin, D. (2011). Uncertainty and risk in wildland fire management. Journal of Environmental Management.

Thompson, M., Bowden, P., Brough, A., Scott, J., & Gilbertson-Day, J. (2015). Application of wildfire risk assessment results to wildfire response planning in the Southern Sierra Nevada, California, USA. Forests.

Thompson, M., Calkin, D., Finney, M., Gebert, K., & Hand, M. (2013). A risk-based approach to wildland fire budgetary planning. Forest Science.

Tolhurst, K., Chong, D., & Duff, T. (2008). Phoenix RapidFire - Bushfire Simulator. Bushfire Cooperative Research Centre, Australia.

Tymstra, C., Bryce, R., & Wotton, B. (2010). Development and structure of Prometheus: The Canadian Wildland Fire Growth Simulation Model. Natural Resources Canada.

Varshney, A. K., & Torra, V. (2023). Recent trends and applications in fuzzy rule-based systems: A survey. International Journal of Fuzzy Systems, 25.

Venn, T., & Calkin, D. (2011). Accommodating non-market values in evaluation of wildfire management in the United States: Challenges and opportunities. International Journal of Wildland Fire.

Wiedinmyer, C., & Neff, J. (2007). Estimates of CO₂ from fires in the United States: implications for carbon management. Carbon Balance and Management.

Yin, H. (2018). Fuzzy-based wildfire risk modeling in spatial-temporal domains. Environmental Modelling & Software.

Zadeh, L. A. (1975). The concept of a linguistic variable and its application to approximate reasoning. Information Sciences.